Acute Logic Analyzer Bus Decode Programming guide

Version: 1.04

Publish: 2014/12/10

內容

BASIC	3

_DECODECFG struct	3

Analysis result	4

How to new a Bus Decodeprokect	5

How to use the Bus Decode(DLL)	7

Bus Decode function	8

int _GetDecodeInfo(int* piMaxCh, char *szName, int iNameBufSize, int** piRptField)	8

int _Analysis(LPDECODECFG lpDecodeCfg, I64**pi64RetBoundary, UINT **piRetData)	9

BOOL _BuildReport(LPDECODECFG lpDecodeCfg, I64 *pi64RetBoundary, UINT *piRetData, int iRetDataSize)	9

BOOL _DataConv(LPDECODECFG lpDecodeCfg, int *piData, char *szBuf, int iBufSize, int *piColorIdx)	10

BOOL _ChannelName(LPDECODECFG lpDecodeCfg, int iChIdx, char *szBuf, int iBufSize)	11

void _DefaultSetting(LPDECODECFG lpDecodeCfg)	12

void _SettingsDialog(LPDECODECFG lpDecodeCfg)	12

PublicLib function	13

Read waveform	13

bool wfInit(TRIDX iBegin, TRIDX iEnd)	13

int wfGetStatus()	13

TRIDX wfGetBeginIdx()	14

TRIDX wfGetEndIdx()	14

bool wfGetData(TRIDX *index, LPBYTE lpb)	14

bool wfGetNext(TRIDX *index, LPBYTE lpb)	14

bool wfEnd()	15

Build report	15

bool rptBegin(int iBoundarySize, bool fFileMode=true)	15

HANDLE rptClose()	15

bool rptSetHeaderColumnStr(int iColumnIdx, char* szHeadStr)	16

bool rptSetColorTable(int iColorIdx, COLORREF ColorFG, COLORREF ColorBG)	16

bool rptBuildHeader()	17

void rptSetColumnBoundary(TRIDX iBoundary)	17

bool rptSetColumnColorIdx(int iColumnIdx, int iColorIdx)	17

bool rptSetColumnData(int iColumnIdx, char * szColumnStr)	18

void rptBuildColumnData()	18

Value translation	18

INT utlValueToStr( DWORD dwValue, int iType, int iDigits, char *szStr, int iBufSize)	18

INT utlValue64ToStr( DWORD dwValueLo, DWORD dwValueHi, int iType, int iDigits, char *szStr, int iBufSize)	19

INT utlValueToStr_Report( DWORD dwValue, int iType, int iDigits, char *szStr, int iBufSize)	20

INT utlValue64ToStr_Report( DWORD dwValueLo, DWORD dwValueHi, int iType, int iDigits, char *szStr, int iBufSize)	21

Update the progress bar	22

void utlInitProgress(int iMin, int iMax, I64 i64Step)	22

void utlSetProgress(I64 i64Current)	22

Other function	23

bool utlGetWorkDir(HWND hWnd, char *szFilePathName)	23

int utlGetErrorCode()	23

int utlGetVersion()	23

# BASIC

PublicLib.lib: there are basic function of Bus Decode, include read waveform and build report. Then LA viewer will display the analysis information.

# _DECODECFG struct

typedef struct _DECODECFG

{

UINT cbSize; 					// size of this structure

int iLaMaxCh; 				// [in] the maximum available channel number.

I64 i64LaSamples;				// [in] the Sample point number

I64 i64LaCsr[DECODE_CSR_CNT];	// [in] the location of cursor A ~F

// 0 : Cursor T, 1 : Cursor A, 2 : Cursor B, ...

int iLaValType;					// [in] the value type (detail as follows)

int iNumOfUsedChannel;			// [in, out] Bus Decode used channel number

//Ex：I2C, iNumOfUsedChannel = 2

//if Bus channel number changed, it should change, too.

HWND hWnd; 				// [in] LA Viewer’s Window Handle

BOOL fRetCode; 				// [out] LA return value of _SettingsDialog() & _DefaultSetting()

//	 true: Setting ok，false: Cancel.

I64 i64SampleRate; 				// [in] Sample Rate

// Bus Decode settings

WORD wFlag; 				// [in, out] Bus decode’s parameter (detail as follows)

int iCh[DECODE_CH_CNT];			// [in, out] channel index (detail as follows)

int iPara[DECODE_PARAM_CNT]; 	// [in, out] Bus decode’s buffer (detail as follows)

} DECODECFG, FAR *LPDECODECFG;

iLaValType:

| Value | Descript |
| --- | --- |
| #define LA_VALUE_TYPE_HEX 0 | Hexadecimal |
| #define LA_VALUE_TYPE_DEC 1 | Decimal |
| #define LA_VALUE_TYPE_OCT 2 | Octal |
| #define LA_VALUE_TYPE_BIN 3 | Binary |
| #define LA_VALUE_TYPE_ASC 4 | ASCII |

wFlag:

Bus Decode’s parameter, initial as 0 when create this bus.

| Value | Descript |
| --- | --- |
| #define WFLAG_USED 0x01 | After create the bus, should set as used. |

iCh[256]:

The channel index, iCh[0] = 5 means analysis only according to channel 5. iCh[0] = 1, iCh[1]=2 means the bus use 2 channel and analysis according to channel 1, 2.

The iCh[] should be modified in setting dialog and the buffer size must less than the value of iLaMaxCh. The max channel number is 64.

iPara[3]:

Bus Decode’s buffer and initial as 0.

# Analysis result

_Analysis(): return the data value(piRetData) and boundary position(pi64RetBoundary) to display on LA viewer.

piRetData (int): the array of data value.

Pi64RetBoundary: the boundary position array.

Every po64RetBoundary can save 32 int data(32bits*32 =1024bits) most.

Ex: the boundary Position 123 has 3 data value.

| Index | Boundary Position (pi64RetBoundary[Index]) | Data Value (piRetData[Index]) |
| --- | --- | --- |
| 0 | 68 | 0x2EA5 |
| 1 | 123 | 0x55AA |
| 2 | 123 | 0x1234 |
| 3 | 123 | 0x7A6D |
| 4 | 234 | 0x85F6 |
| 5 | 345 | 0xD931 |

When LA viewer show waveform, it will keep calling _DataConv() and the _DataConv() should give the string of decode result to show.

# How to new a Bus Decodeprokect

- Open the folder file of DecodeSampleCode .

- Run the CreatePrjWin.exe

- Bus decode name: the project name, the project name cannot use space “_“. If the project name is “MyDecode”, it will new a “AqMyDecode” project

- Bus decode’s title: the name will show on the setting dialog. 
here we take the ”My Bus Decode” for example.

- Click Create button and it will start to create new project and close CreatePrjWin

- Now, we create an AqMyDecode folder in the DecodeSampleCode folder.

- Open the project file in the AqMyDecode folder.

  - VS2008: AqMyDecode.vcproj

  - VS2010: AqMyDecode_VS2010.vcxproj

  - VS2012: AqMyDecode_VS2012.vcxproj

  - VS2013: AqMyDecode_VS2013.vcxproj

- All need file is created.

- Before develop bus decode, you can build the project to check there is no error.

# How to use the Bus Decode(DLL)

After you build the .dll file. Please copy to the C:\Program Files (x86)\Acute\LA\Plugin.

# Bus Decode function

## int _GetDecodeInfo(int* piMaxCh, char *szName, int iNameBufSize, int** piRptField)

Explanation：

Provide the information of Bus decode to LA Viewer.

Parameter：

piMaxCh [out]：

Type：int *

The max. index, -1 means depend on the LA device.

szName [out]：

Type： char*

The name of the Bus Decode, it will show on the Bus Decode list.

iNameBufSize [in]：

Type：int

szName’s buffer size

piRptField [out]：

Type：int **

The array of the column width of report, please padding 0 at last. EX: {100,150,120,0} means there are 3 columns.

return：

return the max channel number can be used, include used and non-used.

## int _Analysis(LPDECODECFG lpDecodeCfg, I64**pi64RetBoundary, UINT **piRetData)

Explanation：

The main analyses for the bus decode. It will provide the boundary/data array until the DLL over.

Parameter：

lpDecodeCfg [in]：

Type：LPDECODECFG

Bus Decode parameter

pi64RetBoundary [out]：

Type：I64 **

Return Boundary Array

piRetData [out]：

Type：UINT **

Return Data Array

Return：

Return Boundary size

## BOOL _BuildReport(LPDECODECFG lpDecodeCfg, I64 *pi64RetBoundary, UINT *piRetData, int iRetDataSize)

Explanation：

After _Analysis(), it will build report by the Boundary/Data Array

Parameter：

lpDecodeCfg [in]：

Type：LPDECODECFG

Bus Decode parameter

pi64RetBoundary [in]：

Type：I64 *

Boundary Array

piRetData [in]：

Type：UINT *

Data Array

iRetDataSize [in]：

Type：int

Boundary/Data Array的size

Return：

Return TRUE

## BOOL _DataConv(LPDECODECFG lpDecodeCfg, int *piData, char *szBuf, int iBufSize, int *piColorIdx)

Explanation：

It will show the string on the waveform .

Parameter：

lpDecodeCfg [in]：

Type：LPDECODECFG

Bus Decode parameter

piData [in]：

Type：int*

Data Value，1st: the number the data in a boundary position, and 32 is maximum.

| Index | 0 | 1 | 2 | 3 | 4 | … | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Content | Length | Data | Data | Data | Data | … | Data |

pszBuf [out]：

Type：char*

The buffer of showing string.

iBufSize [in]：

Type：int

pszBuf size

piColorIdx [out]：

Type：int*

Return the color index(0~15， -1 means no color)

Return：

Return TRUE

## BOOL _ChannelName(LPDECODECFG lpDecodeCfg, int iChIdx, char *szBuf, int iBufSize)

Explanation：

Retrieve the channel name to show on the UI

Parameter：

lpDecodeCfg [in]：

Type：LPDECODECFG

Bus Decode parameter

iChIdx [in]：

Type：int

Channel index.

szBuf [out]：

Type：char*

The buffer of string.

iBufSize [in]：

Type：int

pszBuf size

Return：

Return TRUE

## void _DefaultSetting(LPDECODECFG lpDecodeCfg)

Explanation：

Use for quick Settings, The software will not open Settings Dialog

Parameter：

lpDecodeCfg [in]：

Type：LPDECODECFG

Bus Decode parameter

## void _SettingsDialog(LPDECODECFG lpDecodeCfg)

Explanation：

It will create a setting dialog, set all settings for bus decode.

Parameter：

lpDecodeCfg [in]：

Type：LPDECODECFG

Bus Decode parameter

# PublicLib function

## Read waveform

## bool wfInit(TRIDX iBegin, TRIDX iEnd)

Explanation：

Initial read waveform; call this function before read the waveform.

Parameter：

iBegin [in]：

Type：TRIDX

The start position of the analyzed waveform.

iEnd [in]：

Type：TRIDX

The end position of the analyzed waveform.

Return：

True: initial OK; false: initial fail.

## int wfGetStatus()

Explanation：

Retrieve current state.

Return：

| State | Explanation |
| --- | --- |
| #define WF_NO_SIGNAL 0x01 | No waveform |
| #define WF_FORCE_STOP 0x02 | User had clicked stop button.鍵，應停止分析動作 |

## TRIDX wfGetBeginIdx()

Explanation:

Retrieve the start position of the analyzed waveform.

Return：

The start position of the analyzed waveform.

## TRIDX wfGetEndIdx()

Explanation:

Retrieve the end position of the analyzed waveform.

Return：

The end position of the analyzed waveform

## bool wfGetData(TRIDX *index, LPBYTE lpb)

Explanation:

Retrieve the first edge position and data value of the waveform and the data value.

Parameter：

index [out]

Type：TRIDX*

Retrieve the first edge position of the waveform.

lpb [out]：

Type：LPBYTE

Need 32 bytes buffer to save the data value.

lpb[0] Bit0: Ch0;lpb[0] Bit1: Ch1;lpb[1] Bit0: Ch8，lpb[2] Bit0: Ch16……

It can ue GET_TR_1CH_DATA(Buf, Ch) to retrieve specific channel data.

Retuen:

True: Ok; false: fail.

## bool wfGetNext(TRIDX *index, LPBYTE lpb)

Explanation：

Retrieve next edge position and data value of the waveform.

Parameter：

index [out]

Type：TRIDX*

Retrieve the next edge position of the waveform.

lpb [out]：

Type：LPBYTE

Need 32 bytes buffer to save the data value

lpb[0] Bit0: Ch0;lpb[0] Bit1: Ch1;lpb[1] Bit0: Ch8，lpb[2] Bit0: Ch16……

It can ue GET_TR_1CH_DATA(Buf, Ch) to retrieve specific channel data.

Return：

True: Ok; false: fail.

## bool wfEnd()

Explanation:

It should call this function after read out all waveform.

Return：

True: Ok; false: fail.

## Build report

## bool rptBegin(int iBoundarySize, bool fFileMode=true)

Explanation：

Initializing build report.

Parameter：

iBoundarySize [in]：

Type：int

Boundary array size

fFileMode [in]：

Type：bool

Default: true。

Return：

True: Ok; false: fail.

## HANDLE rptClose()

Explanation：

When report build OK, must call this function.

Return：

Return HANDLE. Keep the handle to display the report. It must be closed before call rptBegin(), avoid memory leak.

## bool rptSetHeaderColumnStr(int iColumnIdx, char* szHeadStr)

Explanation：

The string of the report header.

parameter：

iColumnIdx [in]：

Type：int

The column index of header

szHeadStr [in]：

Type：char*

The string of header

Return：

True: Ok; false: fail.

## bool rptSetColorTable(int iColorIdx, COLORREF ColorFG, COLORREF ColorBG)

Explanation：

Color Table setting, rptSetColumnColorIdx(...) will reference this settings.

Parameter：

iColorIdx [in]：

Type：int

Color Index, start from 0.

ColorFG [in]：

Type：COLORREF

Color of string

ColorBG [in]：

Type：COLORREF

Back ground color

Return：

True: Ok; false: fail.

## bool rptBuildHeader()

Explanation：

After rptSetHeaderColumnStr(), call this function to build header.

Return：

True: Ok; false: fail.

## void rptSetColumnBoundary(TRIDX iBoundary)

Explanation：

The boundary setting。

Parameter：

iBoundary [in]：

Type：TRIDX

The boundary of a row in the report.

## bool rptSetColumnColorIdx(int iColumnIdx, int iColorIdx)

Explanation：

The column color settings

Parameter：

iColumnIdx [in]：

Type：int

The column Index

iColorIdx [in]：

Type：int

The color index, according to the index setting of rptSetColorTable()

Return：

True: Ok; false: fail.

## bool rptSetColumnData(int iColumnIdx, char * szColumnStr)

Explanation：

Set the column data

Parameter：

iColumnIdx [in]：

Type：int

The column index

szColumnStr [in]：

Type：char*

The column string

Return：

True: Ok; false: fail.

## void rptBuildColumnData()

Explanation：

When the column data is ok, call this function to build this column.

## Value translation

## INT utlValueToStr( DWORD dwValue, int iType, int iDigits, char *szStr, int iBufSize)

Explanation：

Convert the value to string

Parameter：

dwValue [in]：

Type：DWORD

The converted value

iType [in]：

Type：int

Value type, use DECODECFG.iLaValType is fine

| Value | Descript |
| --- | --- |
| #define LA_VALUE_TYPE_HEX 0 | Hexadecimal |
| #define LA_VALUE_TYPE_DEC 1 | Decimal |
| #define LA_VALUE_TYPE_OCT 2 | Octal |
| #define LA_VALUE_TYPE_BIN 3 | Binary |
| #define LA_VALUE_TYPE_ASC 4 | ASCII |

iDigits [in]：

Type：int

The bits length of the value

szStr [out]：

Type：char*

The buffer the string

iBufSize [in]：

Type：int

szStr buffer size

Return：

The string length

## INT utlValue64ToStr( DWORD dwValueLo, DWORD dwValueHi, int iType, int iDigits, char *szStr, int iBufSize)

Explanation：

Convert the 64 bits value to string in specific type.

Parameter：

dwValueLo [in]：

Type：DWORD

The low order 32 bits of converted value.

dwValueHi [in]：

Type：DWORD

The high order 32 bits of converted value.

iType [in]：

Type：int

LA Viewer value type, use DECODECFG.iLaValType is fine.

| Value | Descript |
| --- | --- |
| #define LA_VALUE_TYPE_HEX 0 | Hexadecimal |
| #define LA_VALUE_TYPE_DEC 1 | Decimal |
| #define LA_VALUE_TYPE_OCT 2 | Octal |
| #define LA_VALUE_TYPE_BIN 3 | Binary |
| #define LA_VALUE_TYPE_ASC 4 | ASCII |

iDigits [in]：

Type：int

The bits length of the value

szStr [out]：

Type：char*

The buffer the string

iBufSize [in]：

Type：int

szStr buffer size

Return：

The string length

## INT utlValueToStr_Report( DWORD dwValue, int iType, int iDigits, char *szStr, int iBufSize)

Explanation

Convert the value to string in specific type and only for report window.

Parameter：

dwValue [in]：

Type：DWORD

The converted value

iType [in]：

Type：int

LA Viewer value type, use DECODECFG.iLaValType is fine

| 數值 | 說明 |
| --- | --- |
| #define LA_VALUE_TYPE_HEX 0 | Hexadecimal |
| #define LA_VALUE_TYPE_DEC 1 | Decimal |
| #define LA_VALUE_TYPE_OCT 2 | Octal |
| #define LA_VALUE_TYPE_BIN 3 | Binary |
| #define LA_VALUE_TYPE_ASC 4 | ASCII |

iDigits [in]：

Type：int

The bits length of the value

szStr [out]：

Type：char*

The buffer the string

iBufSize [in]：

Type：int

szStr buffer size

Return：

The string length

## INT utlValue64ToStr_Report( DWORD dwValueLo, DWORD dwValueHi, int iType, int iDigits, char *szStr, int iBufSize)

Explanation：

Convert the 64 bits value to string in specific type and only for report window.

參數：

dwValueLo [in]：

Type：DWORD

The low order 32 bits of converted value.

dwValueHi [in]：

Type：DWORD

The high order 32 bits of converted value.

iType [in]：

Type：int

LA Viewer value type, use DECODECFG.iLaValType is fine

| Value | Descript |
| --- | --- |
| #define LA_VALUE_TYPE_HEX 0 | Hexadecimal |
| #define LA_VALUE_TYPE_DEC 1 | Decimal |
| #define LA_VALUE_TYPE_OCT 2 | Octal |
| #define LA_VALUE_TYPE_BIN 3 | Binary |
| #define LA_VALUE_TYPE_ASC 4 | ASCII |

iDigits [in]：

Type：int

The bits length of the value

szStr [out]：

Type：char*

The buffer the string

iBufSize [in]：

Type：int

szStr buffer size

Return：

The string length

## Update the progress bar

Update the progress bar when analyzing and build report.

## void utlInitProgress(int iMin, int iMax, I64 i64Step)

Explanation：

Initialize the progress bar.

Parameter：

iMin [in]：

Type：int

Start percentage, range: 0~1000

iMax [in]：

Type：int

End percentage, range: 0~1000

i64Step [in]：

Type：I64

Update step number.

## void utlSetProgress(I64 i64Current)

Explanation：

Initial the progress bar

Parameter：

i64Current [in]：

Type：I64

Update current progress(Must less than i64Step)

## Other function

## bool utlGetWorkDir(HWND hWnd, char *szFilePathName)

Explanation：

Retrieve the working director of LA Viewer.

Parameter：

hWnd [in]：

Main window handle

szFilePathName [out]：

Retrieve the working director

Return：

True: Ok; false: fail.

## int utlGetErrorCode()

Explanation：

Retrieve the error code

Return：

| Value | Descript |
| --- | --- |
| #define LA_ERR_NONE 0 | None error |
| #define LA_ERR_CFG_NOT_SET 1 | Parameter setting not ok |
| #define LA_ERR_CLASS_CREATE_FAIL 2 | Create class fail |
| #define LA_ERR_RPT_COL_IDX 3 | Column Index is over range |
| #define LA_ERR_RPT_STRING 4 | The string pointer is NULL |
| #define LA_ERR_RPT_COLOR_IDX 5 | Color Index is over range |
| #define LA_ERR_RPT_BUILD_HEAD 6 | Build report header fail |
| #define LA_ERR_NULL_HWND 7 | The handle of main window is NULL |
| #define LA_ERR_SEND_MSG_FAIL 8 | Send Message fail |
| #define LA_ERR_BUF_NULL 9 | The pointer of buffer is NULL |

## int utlGetVersion()

Explanation：

Retrieve the version of PublicLib

Return：

The version of PublicLib

Acute Technology Inc.

www.acute.com.tw

Address：6F-7, #12, Ln. 609, Sec. 5, Chongxin Rd., Sanchong Dist.,

New Taipei City 24159, Taiwan

Tel：+886-2-2999-3275

Fax：+886-2-2999-3276

E-mail: service@acute.com.tw